HIGH-ENERGY LASER FOCUSING LENS @ 1064NM                             LIST S7-08

| ITEM | QTY | PART NO./DESCRIPTION |  |
| --- | --- | --- | --- |
| B. B1 B2 B3 B4 B5 B6 |  | ### Plano-Convex 1064nm Laser Focusing Lens ### Plano-Convex design ### Material: BK 7 glass ### Surface quality: 20-10 scratch/dig(AR coated) ### Diameter: as shown below ### Edge thickness: Te as shown below COATING: ### SURFACE – 1 and -2 : AR Coating ### AR/AR @ 1064nm coating (R < 0.1%) ### DIA: 12.7mm; Te=2mm; EFL = 50mm Ultra-Long Focal Length ### DIA: 25.4mm; Te=3mm; EFL = 1000mm ### DIA: 25.4mm; Te=3mm; EFL = 1500mm ### DIA: 25.4mm; Te=3mm; EFL = 2000mm ### DIA: 25.4mm; Te=3mm; EFL = 2500mm ### DIA: 25.4mm; Te=3mm; EFL = 3000mm | D ### S2 ### Surface 1 & 2 AR @1064nm @ 0o AOI R <0.1% ### S1 |